# Nyquist Sampling Theorem

In digital communication, signals are representations of information that are transmitted from one point to another in a digital format. Nyquist Sampling is a critical theorem that is used to derive the frequency of the signal to reconstruct without aliasing. Aliasing refers to the distortion or unwanted noise that may destroy a signal's integral value.

It was first proposed by Harry Nyquist of Bell Labs in the late 1940s. Harry expressed the Nyquist Sampling Theorem which established the principle of using sampling to convert a continuous analog signal to a digital signal. He produced a sample rule that should be followed to determine appropriate sample rates.

It states that to reconstruct a continuous analog signal from its sampled version accurately, the sampling rate must be at least twice the highest frequency present in the signal. This ensures that there are enough samples taken per unit of time to capture all the details of the original waveform without introducing aliasing.

The Formula for Nyquist Sampling Theorem can be given as

fs​>=2fm​

Where,

fs​ refers to frequency signal

fm​ refers to max frequency

Solved example 1

If the highest frequency component of a signal is 4 kHz, then the minimum sampling frequency is:

fs = 2 × 4 kHz = 8 kHz

So the signal must be sampled at 8 kHz or higher.

Solved example 2

If a signal is sampled at 12 kHz, what maximum signal frequency can be reconstructed?

By Nyquist, fs = 2fmax

Therefore fmax = fs / 2 = 12 kHz / 2 = 6 kHz

## Shannon's Channel Capacity Theorem

In the 1940s, Claude Shannon Developed the Channel Capacity using the Concept of Nyquist and Hartley. He called this the channel capacity Theorem.

Shannon's Channel Capacity theorem States that “the maximum rate at which information can be transmitted over a communication channel of a specified bandwidth in the presence of noise.”

In simple terms, Shannon's channel capacity tells us the theoretical highest limit in which any channel can transmit signals with the presence of noise.

It is given by

C=Blog₂(1+S/N)

Where,

C = channel capacity in bits per second (bps)

B = bandwidth of the channel in hertz (Hz)

S/N = signal-to-noise ratio in linear form

Solved example

Given bandwidth B = 3000 Hz and S/N = 1000

Formula: C = B log2(1 + S/N)

So, C = 3000 × log2(1001)

Since log2(1001) ≈ 9.97, C ≈ 3000 × 9.97 = 29,910 bps

Answer: channel capacity ≈ 29.9 kbps